SYNTHETIC DEMONSTRATION DATA. NOT VALID FOR REAL MACHINE OPERATION.

# MR-2024-08 (Rev A) - Approved

Title: Preventive Maintenance & Calibration Report
Effective Date: 2024-08-01
Machine Model: DEMO-CNC-01

## Section 1: Axis Backlash & Laser Interferometer Verification

Periodic 6-month calibration performed on DEMO-CNC-01 by Lead Maintenance Specialist. Ball screws inspected for pitch error and backlash. Maximum allowable X-axis backlash tolerance is 0.008 mm. Actual measured backlash on X-axis: 0.004 mm (PASS). Y-axis backlash: 0.003 mm (PASS). Z-axis backlash: 0.005 mm (PASS).

## Section 2: Lubrication Specifications & Way Lube

Slideway lubrication system requires Mobil Vactra No. 2 (ISO VG 68 way oil). Inspect reservoir fluid level every morning before shift start. Pneumatic pressure regulator set to 6.0 bar. Spindle vibration analysis registered 0.9 mm/s RMS, well below ISO 10816-1 threshold of 1.8 mm/s.

## Equipment Calibration & Specifications Table

| Machine Parameter | Specification Limit | Measured Calibration | Status |
| --- | --- | --- | --- |
| X-Axis Backlash | Max 0.008 mm | 0.004 mm | PASSED |
| Y-Axis Backlash | Max 0.008 mm | 0.003 mm | PASSED |
| Z-Axis Backlash | Max 0.010 mm | 0.005 mm | PASSED |
| Spindle Vibration RMS | Max 1.8 mm/s | 0.9 mm/s | PASSED |
| Way Lube Type | ISO VG 68 (Vactra 2) | Verified | PASSED |